Single Electricity Market

Change REquest CR290 – VAT AGREEMENT Change VAT CHANGE CALCULATION DETAIL SEM R2.2.0

COPYRIGHT NOTICE

All rights reserved. This entire publication is subject to the laws of copyright. This publication may not be reproduced or transmitted in any form or by any means, electronic or manual, including photocopying without the prior written permission of EirGrid plc and SONI Limited.

DOCUMENT DISCLAIMER

Every care and precaution is taken to ensure the accuracy of the information provided herein but such information is provided without warranties express, implied or otherwise howsoever arising and EirGrid plc and SONI Limited to the fullest extent permitted by law shall not be liable for any inaccuracies, errors, omissions or misleading information contained herein.

Document History

| Version | Date | Author | Comment |
| --- | --- | --- | --- |
| 1.0 | 21st January 2013 | SEMO | Initial Version |

Table of Contents

1	Introduction	3

1.1	Purpose of this document	3

1.2	Scope	3

1.3	Background	3

2	Overview	4

3	Energy Calculations	5

3.1	MEtered Generation and Demand	5

3.2	Cross Border Energy Export Proportions	5

3.3	Cross Border Energy Supply Proportions	6

4	Capacity Calculations	7

4.1	Capacity PAyments and Charges	7

4.2	Cross Border Capacity Export Proportions	7

4.3	Cross Border Energy Supply Proportions	8

5	Spreadsheet Calculation Example	9

6	Invoicing	9

7	Cutover	9

8	Glossary of terms	10

New Terminology	10

# Introduction

## Purpose of this document

This document is intended to give a detailed description of the calculations involved in the implementation of CR290 – VAT Agreement Change.

Additional information in relation to this change can be found in the following documents available on the SEMO website:

- the Final Recommendation Report FRR_03._12 and

- the Market Participant Update Document (MPUD Market Interface Volume 2 Reporting - SEM R2.2.0).

## Scope

Only calculations that take place in relation to CR290 will be included in this document

## Background

The issues with the current VAT Agreement that was issued jointly by both Revenue Authorities arose in mid 2011 when an ROI company expressed a wish to join the SEM as an Interconnector Unit (Generator) trading on the Moyle Interconnector. The provisions in the Trading & Settlement Code and SEM VAT Agreement (agreed by VAT authorities in both jurisdictions) require that all units must be VAT registered in the Jurisdiction that the Unit is registered. As a result, they were unable to get a VAT registration in NI.

Although the SEM systems correctly reflect the current VAT agreement; this was still preventing certain companies from joining the SEM. It was clear that a revision was required to the VAT Agreement to reflect the broader interest in joining the SEM and to reflect the correct VAT law associated with this.

The VAT Authorities have, following discussions with the Market Operator, amended the existing VAT Agreement to include an Addendum that satisfies the EU VAT legislation requirements. A change to the Central Market Systems is necessary to implement these changes and a Change Request (CR290) was raised as a result. This document describes in detail, the calculations involved in the implementation of CR290 in the Central Market Systems.

CR290 is expected to be implemented as part of the SEM R2.2.0 release which is scheduled for deployment on May 10th 2013.

# Overview

For units to be part of the SEM, they must be located in Northern Ireland or the Republic of Ireland.

With the advent of the changes to VAT agreement (and SEM R2.2.0), it will be possible for Generator Participants to be VAT registered anywhere in the world. For the purposes of the application of VAT, this is broken down into four categories.

ROI – VAT registered in the Republic of Ireland

UK – VAT registered in the United Kingdom

EU – VAT registered in the EU but outside the Republic of Ireland and United Kingdom

Non-EU – VAT registered outside the EU

This means that there are 8 Participant\Unit combinations possible for Generator Participants.

| Participant VAT Jurisdiction | Unit Jurisdiction |
| --- | --- |
| ROI | ROI |
| UK | ROI |
| EU | ROI |
| Non-EU | ROI |
| ROI | NI |
| UK | NI |
| EU | NI |
| Non-EU | NI |

Supplier Participants must be registered in the same location as their units so the number of combinations reduces to 2. This is unchanged from the previous version of VAT agreement.

| Participant VAT Jurisdiction | Unit Jurisdiction |
| --- | --- |
| ROI | ROI |
| UK | NI |

VAT is applied on invoice amounts based on the VAT Jurisdiction of both sides of the energy transaction.

As the SEM operates as a pool, a calculation is required to determine the proportion of the pool in each jurisdiction which is:

- Generated by Participants VAT registered in that jurisdiction

- Generated by Participants VAT registered in EU (but outside the jurisdiction)

- Generated by Participants VAT registered outside the EU

- Consumed by Participants VAT registered in that jurisdiction

- Consumed by Participants VAT registered in EU (but outside the jurisdiction)

In system terms, these proportions are called

- CBESP\CBCSP – Cross Border Energy\Capacity Supplied Proportion

- CBESPEU \ CBCSPEU – Cross Border Energy\Capacity Supplied Proportion

- CBESPNEU\ CBCSPNEU – Cross Border Energy\Capacity Supplied Proportion

- CBEEPI\CBCEPI – Inverse Cross Border Energy\Capacity Export Proportion

- CBEEP\CBCEP – Cross Border Energy\Capacity Export Proportion

Proportions 1), 2) and 3) will replace the existing proportions CBEIPI\CBCIPI and CBEIP\CBCIP. These proportions are used to split amounts on Supplier Invoices according to the VAT jurisdiction where the energy consumed is deemed to have been generated.

Proportions 4) and 5) currently exist in the systems and no change is expected to their calculation. These proportions are used to split amounts on Generator Invoices according to the VAT jurisdiction where the generated energy is deemed to have been consumed.

# Energy Calculations

## MEtered Generation and Demand

- MGLFuh = Loss-adjusted Meter Generation for Generator unit u in trading period h

- SNDLFvh = Loss- adjusted Settlement Net Demand for Supplier unit v in trading period h

- SWPMGLFpb = Sum(MGLFuh) for all Generator units u registered to participant account p and trading period h in Energy billing week b.

- SWPNDLFpb = Sum(SNDLFvh) for all Supplier units v registered to participant account p and trading period h in Energy billing week b.

- SWJMGLF_ROIb = Sum(SWPMGLFpb)  for all participant accounts p that are registered in ROI VAT Jurisdiction

- SWJMGLF_NIb = Sum(SWPMGLFpb)  for all participant accounts p that are registered in UK VAT Jurisdiction

- SWJNDLF_ROIb = Sum(SWPNDLFpb)  for all participant accounts p that are registered in ROI VAT Jurisdiction

- SWJNDLF_NIb = Sum(SWPNDLFpb)  for all participant accounts p that are registered in UK VAT Jurisdiction

- SWJMGLF_SJ_ROIpb = Sum(SWPMGLFpb) for all participant accounts p where both participants and generators are registered in ROI VAT Jurisdiction.

- SWJMGLF_SJ_NIpb = Sum(SWPMGLFpb) for all participant accounts p where both participants and generators are registered in NI VAT Jurisdiction.

- SWJMGLF_OJ_ROIpb = Sum(SWPMGLFpb) for all participant accounts p that are registered in UK VAT Jurisdiction. and have their generators registered in ROI Jurisdiction.

- SWJMGLF_OJ_NIpb = Sum(SWPMGLFpb) for all participant accounts p that are registered in ROI VAT Jurisdiction. and have their generators registered in UK Jurisdiction.

- SWJMGLF_EU_ROIpb = Sum(SWPMGLFpb) for all participant accounts p that are registered in EU VAT Jurisdiction. (excluding ROI and UK) and have their generators registered in ROI Jurisdiction.

- SWJMGLF_EU_NIpb = Sum(SWPMGLFpb) for all participant accounts p that are registered in EU VAT Jurisdiction. (excluding ROI and UK) and have their generators registered in UK Jurisdiction.

- SWJMGLF_NEU_ROIpb = Sum(SWPMGLFpb) for all participant accounts p that are registered in Non-EU VAT Jurisdiction and have their generators registered in ROI Jurisdiction.

- SWJMGLF_NEU_NIpb = Sum(SWPMGLFpb) for all participant accounts p that are registered in Non-EU VAT Jurisdiction and have their generators registered in UK Jurisdiction.

## Cross Border Energy Export Proportions

- CBEEP_ROIb = IF((SWJMGLF_ROIb – SWPNDLF_ROIb) >0, (SWJMGLF_ROIb – SWPNDLF_ROIb) / SWJMGLF_ROIb,0)

- CBEEPI_ROIb = 1 - CBEEP_ROIb

- CBEEP_NIb = = IF((SWJMGLF_NIb – SWPNDLF_NIb) >0, (SWJMGLF_NIb – SWPNDLF_NIb) / SWJMGLF_Nib,0)

- CBEEPI_NIb = 1 - CBEEP_NIb

## Cross Border Energy Supply Proportions

- TSJG_ROI =	   (SWJMGLF_SJ_ROIb

- + SWJMGLF_OJ_ROIb

- + SWJMGLF_EU_ROIb

- + SWJMGLF_NEU_ROIb)

- * CBEEPI_ROIb

- + (SWJMGLF_SJ_Nb

- + SWJMGLF_OJ_Nib

- + SWJMGLF_EU_Nib

- + SWJMGLF_NEU_NIb)

- * CBEEP_Nib

- TSJG_NI =	   (SWJMGLF_SJ_NIb

- + SWJMGLF_OJ_NIb

- + SWJMGLF_EU_NIb

- + SWJMGLF_NEU_NIb)

- * CBEEPI_NIb

- + (SWJMGLF_SJ_ROI

- + SWJMGLF_OJ_ROib

- + SWJMGLF_EU_ROib

- + SWJMGLF_NEU_ROIb)

- * CBEEP_ROib

- CBESP_ROIb= (SWJMGLF_SJ_ROIb * CBEEPI_ROIb +

- SWJMGLF_OJ_NIb * CBEEP_NIb)

- /TSJG_ROIb

- CBESPEU_ROIb = ((SWJMGLF_OJ_ROIb + SWJMGLF_EU_ROIb) * CBEEPI_ROIb +

- (SWJMGLF_SJ_NIb + SWJMGLF_EU_NIb) * CBEEP_NIb)

- /TSJG_ROIb

- CBESPNEU_ROIb= (SWJMGLF_NEU_ROIb * CBEEPI_ROIb +

- SWJMGLF_NEU_NIb * CBEEP_NIb)

- /TSJG_ROIb

- CBESP_NIb= (SWJMGLF_SJ_NIb * CBEEPI_NIb +

- SWJMGLF_OJ_ROIb * CBEEP_ROIb)

- /TSJG_NIb

- CBESPEU_NIb = ((SWJMGLF_OJ_NIb + SWJMGLF_EU_NIb) * CBEEPI_NIb +

- (SWJMGLF_SJ_ROIb + SWJMGLF_EU_ROIb) * CBEEP_ROIb)

- /TSJG_NIb

- CBESPNEU_NIb= (SWJMGLF_NEU_NIb * CBEEPI_NIb +

- SWJMGLF_NEU_ROIb * CBEEP_ROIb)

- /TSJG_Nib

# Capacity Calculations

## Capacity PAyments and Charges

- SMPCPpc = Sum(Capacity Payments) for all Generator units u registered to participant account p and trading period h in Capacity billing month c.

- SMPCCpc = Sum(Capacity Payments) for all Supplier units v registered to participant account p and trading period h in Capacity billing month c.

- CPPJ_ROIc = Sum(SMPCPpc)  for all participant accounts p that are registered in ROI VAT Jurisdiction

- CPPJ_NIc = Sum(SMPCPpc)  for all participant accounts p that are registered in UK VAT Jurisdiction

- CPCJ_ROIc = Sum(SMPCCpc)  for all participant accounts p that are registered in ROI VAT Jurisdiction

- CPCJ_NIc = Sum(SMPCCpc)  for all participant accounts p that are registered in UK VAT Jurisdiction

- SMJCP_SJ_ROIpc = Sum(SMPCPpc) for all participant accounts p where both participants and generators are registered in ROI VAT Jurisdiction.

- SMJCP_SJ_NIpc = Sum(SMPCPpc) for all participant accounts p where both participants and generators are registered in NI VAT Jurisdiction.

- SMJCP_OJ_ROIpc = Sum(SMPCPpc) for all participant accounts p that are registered in UK VAT Jurisdiction and have their generators registered in ROI Jurisdiction.

- SMJCP_OJ_NIpc = Sum(SMPCPpc) for all participant accounts p that are registered in ROI VAT Jurisdiction and have their generators registered in UK Jurisdiction.

- SMJCP_EU_ROIpc = Sum(SMPCPpc) for all participant accounts p that are registered in EU VAT Jurisdiction (excluding ROI and UK) and have their generators registered in ROI Jurisdiction.

- SMJCP_EU_NIpc = Sum(SMPCPpc) for all participant accounts p that are registered in EU VAT Jurisdiction (excluding ROI and UK) and have their generators registered in UK Jurisdiction.

- SMJCP_NEU_ROIpc = Sum(SMPCPpc) for all participant accounts p that are registered in Non-EU VAT Jurisdiction and have their generators registered in ROI Jurisdiction.

- SMJCP_NEU_NIpc = Sum(SMPCPpc) for all participant accounts p that are registered in Non-EU VAT Jurisdiction and have their generators registered in UK Jurisdiction.

## Cross Border Capacity Export Proportions

- CBCEP_ROIc = IF((CPPJ_ROIb – CPCJ_ROIb) >0, (CPPJ_ROIb – CPCJ _ROIb) / CPPJ_ROIb,0)

- CBCEPI_ROIc = 1 - CBCEP_ROIb

- CBCEP_NIc = = IF((CPPJ_NIb – CPCJ _NIb) >0, (CPPJ_NIb – CPCJ _NIb) / CPPJ_Nib,0)

- CBCEPI_NIc = 1 - CBCEP_NIb

## Cross Border Energy Supply Proportions

- TSJCP_ROI =	   (SMJCP_SJ_ROIb

- + SMJCP_OJ_ROIb

- + SMJCP_EU_ROIb

- + SMJCP_NEU_ROIb)

- * CBCEPI_ROIb

- + (SMJCP_SJ_Nb

- + SMJCP_OJ_Nib

- + SMJCP_EU_Nib

- + SMJCP_NEU_NIb)

- * CBCEP_Nib

- TSJCP_NI =	   (SMJCP_SJ_NIb

- + SMJCP_OJ_NIb

- + SMJCP_EU_NIb

- + SMJCP_NEU_NIb)

- * CBCEPI_NIb

- + (SMJCP_SJ_ROI

- + SMJCP_OJ_ROib

- + SMJCP_EU_ROib

- + SMJCP_NEU_ROIb)

- * CBCEP_ROib

- CBCSP_ROIb= (SMJCP_SJ_ROIb * CBCEPI_ROIb +

- SMJCP_OJ_NIb * CBCEP_NIb)

- /TSJCP_ROIb

- CBCSPEU_ROIb = ((SMJCP_OJ_ROIb + SMJCP_EU_ROIb) * CBCEPI_ROIb +

- (SMJCP_SJ_NIb + SMJCP_EU_NIb) * CBCEP_NIb)

- /TSJCP_ROIb

- CBCSPNEU_ROIb= (SMJCP_NEU_ROIb * CBCEPI_ROIb +

- SMJCP_NEU_NIb * CBCEP_NIb)

- /TSJCP_ROIb

- CBCSP_NIb= (SMJCP_SJ_NIb * CBCEPI_NIb +

- SMJCP_OJ_ROIb * CBCEP_ROIb)

- /TSJCP_NIb

- CBCSPEU_NIb = ((SMJCP_OJ_NIb + SMJCP_EU_NIb) * CBCEPI_NIb +

- (SMJCP_SJ_ROIb + SMJCP_EU_ROIb) * CBCEP_ROIb)

- /TSJCP_NIb

- CBCSPNEU_NIb= (SMJCP_NEU_NIb * CBCEPI_NIb +

- SMJCP_NEU_ROIb * CBCEP_ROIb)

- /TSJCP_NIb

# Spreadsheet Calculation Example

A sample calculation can be found in the following spreadsheet:

# Invoicing

As Supplier participants must still register in the same jurisdiction as their units, all generation is deemed to be sold to participants in ROI and NI. As such, no change is expected for Generator Invoice amounts and the current Cross Border Import Proportions will be used.

E.g.  A charge ENPEX will still be split into ENPEX_N and ENPEX_Z representing local and cross border trades respectively.

As generator Participants can register in a different jurisdiction to their units, values must be calculated which define the proportion of a suppliers consumption which is derived from each VAT jurisdiction (Local, EU and Non-EU).

E.g.  A charge ENCEX will now be split into three invoice amounts, ENCEX_W, ENCEX_EU and ENCEX_NEU, representing supplied amounts generated locally, in the EU and outside the EU respectively.

No changes to Energy or Capacity statements are expected as part of this change.

Section 3 details the calculation of each of Supplier proportions outlined above.

# Cutover

This change is expected to be implemented as part of the release SEM R2.2.0 which is scheduled to take place on May 10th 2013.

Any invoices issued after this date will be in the new SEM R2.2.0 format. This includes resettlement invoices.

However, the new calculations (CBESP, CBESPEU, CBESPNEU etc.) will only apply to billing periods after the effective date of SEM R2.2.0.

The normal Energy billing week runs from a Sunday to a Saturday.

As such, the calculation changes affecting Energy Invoicing are expected to become effective as part of the first Energy invoice run relating to the Energy billing week starting on the first Sunday after the release, currently May 12th 2013.

The normal Capacity billing month runs from the first to last day of each calendar month

As such, the calculation changes affecting Capacity Invoicing are expected to become effective as part of the first Capacity invoice run relating to the Capacity billing month starting on the first day of the month after the release, currently June 1st 2013.

All newly issued invoices relating to periods before the effective dates of the release will continue to use the old calculations (CBEIP, CBEEP) but will appear in the new format.

# Glossary of terms

# New Terminology

| Term | Description |
| --- | --- |
| CBCEP | Cross Border Capacity Export Proportion – The proportion of capacity deemed to have been exported from a jurisdiction |
| CBCSP | Cross Border Capacity Supplied Proportion – The proportion of supplied energy in a jurisdiction deemed to have been generated from participants registered within that same jurisdiction. |
| CBCSPEU | Cross Border Capacity Supplied Proportion – The proportion of supplied energy in a jurisdiction deemed to have been generated from participants registered within the EU (excluding the current jurisdiction) |
| CBCSPNEU | Cross Border Capacity Supplied Proportion – The proportion of supplied energy in a jurisdiction deemed to have been generated from participants registered outside the EU |
| CBEEP | Cross Border Energy Export Proportion – The proportion of energy deemed to have been exported from a jurisdiction |
| CBESP | Cross Border Energy Supplied Proportion – The proportion of supplied energy in a jurisdiction deemed to have been generated from participants registered within that same jurisdiction. |
| CBESPEU | Cross Border Energy Supplied Proportion – The proportion of supplied energy in a jurisdiction deemed to have been generated from participants registered within the EU (excluding the current jurisdiction) |
| CBESPNEU | Cross Border Energy Supplied Proportion – The proportion of supplied energy in a jurisdiction deemed to have been generated from participants registered outside the EU |
| Generator Unit | This term covers both generator and interconnector units as defined by the Trading & Settlement Code |
| Loss-adjusted | Value after loss-factors have been applied |
| MGLF | Loss-adjusted Meter Generation |
| Participant account | As per the Trading & Settlement Code |
| Participant VAT Jurisdiction | Physical location in which a unit is registered Possible Values: ROI UK EU Non-EU |
| SNDLF | Loss- adjusted Settlement Net Demand |
| TSJCP | Total Supplied Jurisdictional Capacity Payments – Total capacity payments made to participants registered in the jurisdiction |
| TSJG | Total Supplied Jurisdictional Generation – Total generation which is supplied to participants registered in the jurisdiction |
| Unit Jurisdiction | Physical location in which a unit is registered Possible Values: ROI NI |